Task Risk Assessment

Task: Bonfires										Date: 27/01/2026

This risk assessment will be updated annually following a review or whenever further risks or control measures are identified.

Prior to the task taking place the task leader will assess the task and give a risk assessment briefing to all staff and volunteers to include use of tools, site conditions, manual handling etc.

| PPE required to carry out this task: | Sturdy footwear, steel toecaps if required. Heavy duty gloves |
| --- | --- |
| Training required to carry out this task: | In-house training |
| Can this task be carried out by a lone worker: | Yes |
| Minimum level of first aid required to carry out this task: (EFAW, FAAW, FAAW+F) | Emergency first aid 1:50 |

| HAZARD Potential Harm | PEOPLE AT RISK | EXISTING CONTROL METHODS Staff and volunteers instructed or trained to: | FURTHER ACTIONS REQUIRED | CORRECTIVE ACTIONS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | WHO | WHEN | COMPLETED |
| Loading, transporting, unloading and use of hand tools and materials: Muscular strain / injury | Staff & volunteers | Staff and volunteers to adhere to guidance in the hand tools risk assessment. Staff and volunteers to adhere to manual handling risk assessment. Wear suitable footwear with good grip, and protected toecaps if carrying heavy objects e.g. blocks for the fire platform. |  |  |  |  |
| Fire: Singes, burns from sparks intense heat and flames | Staff, volunteers & public | Ensure appropriate location of fire, including appropriate access and distance from combustible material. Assess wind speed and likelihood of a fire spreading before lighting. Do not use accelerants to start fire. Maintain fire at appropriate size for the site / task. Use signage and barrier tape as required to keep members of the public at a safe distance from the fire. Maintain a clear area around the fire. Use pitch forks to load material onto fire from a safe distance. Maintain a safe working distance of 2x tool length. Ensure water is available if required. Never leave unattended unless burnt down to embers and cordoned off to prevent access. |  |  |  |  |
| Fire: Smoke inhalation | Staff & volunteers | All staff and volunteers to take regular breaks away from smoke. Take care when lighting the fire to not inhale any smoke. |  |  |  |  |
| Moving vegetation: Cuts/strain injury, puncture wounds from thorns | Staff & volunteers | Group advised to not exceed their personal capabilities. Wear heavy duty gloves. Follow hand tools risk assessment. |  |  |  |  |
| Fatigue: Repetitive task | Staff & volunteers | Group to take regular breaks. Individuals advised to work at own pace and take breaks as required. |  |  |  |  |

The task covered by this risk assessment should only be carried out by a member of staff or volunteer who is trained and authorised to do so. If you have any medical condition which might increase the likelihood of an accident or injury resulting from carrying out this task then you must discuss this with and be authorised by your line manager, before you attempt the task.

|  | Assessment date | Assessor | Position |
| --- | --- | --- | --- |
| Initial assessment undertaken | Nov-2009 | Alison Vallance | Neighborhood Wildlife Warden |
| Review 7 Completed: | 21/12/2016 | Lucy Tomkinson | Community Reserves Warden |
| Review 8 Completed: | 13/09/2017 | George Cooper | Land Management Apprentice |
| Review 9 Completed: | 11/12/2018 | Chris Parker | Head of Land Management |
| Review 10 Completed: | 08/10/2019 | Chris Parker | Head of Land Management |
| Review 11 Completed: | 12/09/2021 | Tim Read | Senior Warden |
| Review 12 Completed: | 13/11/2024 | Tim Read | Countryside Manager |
| Review 13 completed: | 27/01/2026 | Tim Read | Countryside Manager |