Task Risk Assessment

Task: Log Splitting									Date: 27/01/2026

This risk assessment will be updated annually following a review or whenever further risks or control measures are identified.

Prior to the task taking place the task leader will assess the task and give a risk assessment briefing to all staff and volunteers to include use of tools, site conditions, manual handling etc.

| PPE required to carry out this task: | - Steel toe-capped boots - Gloves - Eye protection if striking metal with metal. |
| --- | --- |
| Training required to carry out this task: | In-house |
| Can this task be carried out by a lone worker: | No |
| Minimum level of first aid required to carry out this task: (EFAW, FAAW, FAAW+F) | EFAW 1:50 |

| HAZARD Potential Harm | PEOPLE AT RISK | EXISTING CONTROL METHODS Staff and volunteers instructed or trained to: | FURTHER ACTIONS REQUIRED | CORRECTIVE ACTIONS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | WHO | WHEN | COMPLETED |
| Loading, transporting, using and unloading tools: Cuts/strain injury from repeated lifting, twisting and bending e.g. muscle strain | Staff, volunteers & bystanders | Follow Manual Handling Risk Assessment. Load vehicle blades first for safe unloading. |  |  |  |  |
| Unsafe tool: Axe head or sledgehammer coming loose, flying off and causing injury | Operator, bystanders | Prior to every use, check that the axe/sledgehammer is in good condition and fit for purpose. Do not use equipment if compromised. |  |  |  |  |
| Using axe/sledgehammer: Laceration or bruise injury, muscular injury, skeletal injury, flying debris. | Operator, bystanders | Steel toe-capped boots must always be worn. Keep area in-front of axe swing completely clear. Logs must be split on a stable block and on even and firm ground. Keep work area clear of trip hazards. Demonstrate good posture to minimise muscular injury. Ensure good grip with either latex-coated gloves or bare-hands. Do not strike metal on metal unless necessary; wear eye protection and increase safe working distance to 10m if so. Check condition of metal tools and do not use if showing cracks/fractures. |  |  |  |  |
| Flying debris: Cuts/ other injury from flying debris | Operator, bystanders | Ensure all are working to safe-working distance: - at least 5 metres between operators and bystanders/co-workers. If striking metal on metal ensure operators are wearing eye protection and adjust safe working distance to 10m. |  |  |  |  |
| Using a log grenade: Laceration injury, skeletal injury. | Operator, bystanders | Only use a log grenade if necessary. Wear eye protection and increase safe working distance to 10m. Reposition log grenade if placed in area with high tension (it rebounds on impact). |  |  |  |  |
| Operator Fatigue: | Operator | Operator to take regular breaks as required. |  |  |  |  |

The task covered by this risk assessment should only be carried out by a member of staff or volunteer who is trained and authorised to do so. If you have any medical condition which might increase the likelihood of an accident or injury resulting from carrying out this task then you must discuss this with and be authorised by your line manager, before you attempt the task.

|  | Assessment date | Assessor | Position |
| --- | --- | --- | --- |
| Assessment undertaken on: | December 2012 | Chris Parker | Warden |
| Review 1 completed: | 06/02/2013 | Andy Johnson | Farm Manager |
| Review 2 completed: | 06/03/2014 | Luisa Boheimer | Warden |
| Review 3 completed: | 28/04/2015 | Steve Wrigley | Warden |
| Review 4 completed: | 25/10/2016 | Steve Wrigley | Warden |
| Review 5 completed: | 08/08/2017 | George Cooper | Warden |
| Review 6 completed: | 11/12/2018 | Tim Read | Senior Warden |
| Review 7 completed: | 08/10/2019 | Tim Read | Senior Warden |
| Review 8 completed: | 02/09/2021 | Tristan Leslie | Ranger |
| Review 9 completed: | 03/09/2024 | Tim Read | Countryside Manager |
| Review 10 completed: | 27/01/2024 | Tim Read | Countryside Manager |